# Workload and Cluster Requirements

Engineering requirements for CB-2026-041

| Owner | Jonas Reed, Principal ML Systems Engineer |
| --- | --- |
| Version | 1.3 |
| Issued | August 4, 2026 |
| Scope | 1,024-GPU firm deployment |

Synthetic matter: This benchmark document describes a fictional buyer and workload.

# 1. Workload profile

The cluster will run sustained multimodal model pretraining, followed by fine-tuning and evaluation. Training jobs may occupy the full cluster for several days. Checkpoint recovery, collective performance, and predictable maintenance windows matter more than burst elasticity.

# 2. Compute and software

| Requirement | Minimum | Preference or note |
| --- | --- | --- |
| Accelerators | 1,024 NVIDIA B200 SXM GPUs | No prior-generation substitution |
| Node shape | 128 dedicated HGX B200 nodes, eight GPUs each | Homogeneous node fleet |
| Virtualization | No shared or fractional GPU service | Buyer may use its own scheduler |
| Software | Buyer image compatible with CUDA 13 toolchain | Provider supplies validated driver and firmware matrix |
| Configuration change | Northstar written consent | Applies to GPU generation and material node changes |

# 3. Fabric, ingress, and storage

| Area | Requirement | Evidence expected |
| --- | --- | --- |
| Scale-up/scale-out fabric | At least one 400 Gb/s endpoint per GPU; dual-rail 800 Gb/s preferred | Topology, port map, oversubscription statement, and burn-in results |
| External ingress | At least 200 Gb/s committed; 400 Gb/s preferred | Carrier order and test result |
| Shared storage | 6 PiB usable | Capacity and protection-policy statement |
| Read throughput | 1.2 TB/s aggregate sustained | Representative benchmark |
| Write throughput | 800 GB/s aggregate sustained | Representative benchmark |

# 4. Availability and maintenance

Cluster availability: At least 99.5% per calendar month, measured across accepted GPUs

Planned maintenance: No more than eight hours per month

Notice: At least seven days before planned maintenance

Incidents: Severity classification, status communications, root-cause analysis, and corrective action for material events

# 5. Acceptance

Burn-in: 72 continuous hours after each tranche is ready

Inventory: Serial-number and node inventory attestation before testing

Performance: NCCL benchmark against an agreed reference result

Open item: The reference cluster, workload, and passing threshold must be agreed before signing

# 6. Security and location

Data residency: United States only

Controls: Current SOC 2 Type II report

Encryption: Customer-managed keys for data at rest where applicable

Logging: Exportable administrative and workload-access logs

Physical access: Controlled GPU-hall access with named-person access records